_______________________________________________________________________________________________________________

SECTION 1: Identification of the substance/mixture and of the company/undertaking

  - Product identifier:

METHYL METHACRYLATE with DIMETHYL TOLUIDIN

Monomer mixture based on methacrylic acid esters  (containing activator)

Substances

- Methyl methacrylate 60-100% (CAS 80-62-6)(EC 201-297-1)(REACH 01-2119452498-28)

- 2-Hydroxyethyl methacrylate 15-40% (CAS 868-77-9)(EC 212-782-2)(REACH 01-2119490169-29)

- N,N-Dimethyl-p-toluidine 1-5% (CAS 99-97-8)(EC 202-805-4)(REACH 01-2119937766-23)

Trade name / designation	Methyl Methacrylate

Hazard components for labelling

- Methyl methacrylate 60-100%

- 2-Hydroxyethyl methacrylate 15-40%

- N,N-Dimethyl-p-toluidine 1-5%

  - Relevant identified uses of the substance or mixture and uses advised against

Relevant identified uses:

Adhesive in application of hoof care products

Uses advised against:

None known

1.3	Details of the supplier of the safety data sheet:

Supplier:

Giltspur Scientific Ltd

6-8 Avondale Industrial Estate

Ballyclare

Northern Ireland

BT39 9AU

+44 (0) 2893 322040

info@giltspurscientific.com

1.4	EMERGENCY TELEPHONE NUMBER:

Evonik							Telephone - +49 61151 1801

Chemische Fabrik						Emergency - +49 6151 18 4342

d-64275 Darmstadt

_______________________________________________________________________________________________________________

SECTION 2:	Hazards identification

2.1.	Classification of the substance or mixture:

Classification according to Regulation (EC) No 1272/2008 [CLP]

H225 Flam. Liq., H315 Skin Irrit., H317 Skin Sens.

Additional information:

# Full text of H- phrases: see SECTION 16.

__________________________________________________________________________________________________________

  - Label elements

Labelling according to Regulation (EC) No 1272/2008 [CLP/GHS]

Hazard statements:

H225 Highly flammable liquid and vapour

H332 Harmful if inhaled

H312 Harmful in contact with skin

H302 Harmful if swallowed

H315 Causes skin irritation

H317 May cause an allergic skin reaction

H319 Causes serious eye irritation

H335 May cause respiratory irritation

Precautionary statements:

P210 Keep away from heat/sparks/open flames/hot surfaces — No smoking

P262 Do not get in eyes, on skin, or on clothing

P280 Wear protective gloves (or barrier cream) and eye protection

P281 Use personal protective equipment as required

P233 Keep container tightly closed

P260 Do not breathe vapours

P271 Use only outdoors or in a well-ventilated area

P285 In case of inadequate ventilation wear respiratory protection

______________________________________________________________________________________________________________

2.3	Other hazards

Polymerisation with heat evolution may occur in the presence of radical forming substances (eg peroxides), reducing substances, and/or heavy metal ions

___________________________________________________________________________________

SECTION 3.	Composition/information on ingredients

3.1	Substances

Not applicable

3.2	Mixtures

Description of the mixture:

Solution of Methyl methacrylate, 2-Hydroxyethyl methacrylate and N,N-Dimethyl-p-toluidine

Hazardous ingredients

| Substance name | CAS No. | EC No. | REACH No. | Concentration | Classification according Regulation (EC) No. 1272 [CLP] |
| --- | --- | --- | --- | --- | --- |
| Methyl methacrylate 2-Hydroxyethyl methacrylate N,N-Dimethyl-p-toluidine | 80-62-6 868-77-9 99-97-8 | 201-297-1 212-782-2 202-805-4 | 01-2119452498-28 01-2119490169-29 01-2119937766-23 | 60-100% 15-40% 1-5% | H225, H315, H317, H335, H370 H225, H302, H315, H317, H319, H335, H351 H301, H330, H331, H351, H373, H412 |

Additional information:

Full text of H-phrases: see SECTION 16.

_________________________________________________________________________________________________________________

SECTION 4:	First aid measures

4.1	Description of first aid measures

General information

Remove soiled, soaked clothing immediately

Medical treatment is necessary if symptoms occur which are obviously caused by skin or eye contact with the product or by inhalation of its vapours.

Following inhalation

Remove the casualty into fresh air and keep him/her calm

Following skin contact

In case of contact with the skin wash off immediately with soap and water. If skin irritation occurs, seek medical attention.

Following eye contact

In case of contact with eyes rinse thoroughly with plenty of water and seek medical advice.

Following ingestion

Summon medical assistance immediately.

4.2	Most important symptoms and effects, both acute and delayed

Irritating to eyes, respiratory system and skin

May cause sensitisation by skin contact

Harmful by inhalation

  - Indication of any immediate medical attention and special treatment needed

Summon medical assistance immediately following ingestion

Medical treatment is necessary if symptoms occur which are obviously caused by skin or eye contact with the product or by inhalation of its vapours

____________________________________________________________________________________________________________________________

SECTION 5:	Firefighting measures

5.1	Extinguishing media
	Suitable extinguishing media

Foam

Dry powder

Carbon dioxide

Water SPRAY jet

Unsuitable extinguishing media

Do NOT extinguish with full water jet

5.2	Special hazards arising from the substance or mixture

May be released in case of fire: carbon monoxide, carbon dioxide, organic products of decomposition

5.3	Advice for fire-fighters

In the event of fire, cool the endangered containers with water.

Wear self-contained breathing apparatus.

Do not discharge into the drains/ surface waters/ ground water.

____________________________________________________________________________________________________________

SECTION 6:	Accidental release measures

6.1	Personal precautions, protective equipment and emergency procedures

For non-emergency personnel

Ensure adequate ventilation

P284 Wear respiratory protection

P281 Use personal protective equipment as required

For emergency responders

Ensure adequate ventilation

P284 Wear respiratory protection

P281 Use personal protective equipment as required

  - Environmental precautions

Do not discharge into the drains/ surface waters/ ground water

  - Methods and material for containment and cleaning up

Larger quantities – remove mechanically (by pumping)

Smaller quantities and/or residues – absorb with absorbent material eg. sand or sawdust

Dispose of in accordance with regulations

6.4	Reference to other sections

For personal protection see section 8.

___________________________________________________________________________________________________________________________

SECTION 7:	Handling and storage

7.1	Precautions for safe handling

P233 Keep containers tightly closed.

P271 Use only outdoors or in a well-ventilated area

P285 In case of inadequate ventilation wear respiratory protection

P210 Keep away from heat/sparks/open flames/hot surfaces — No smoking

P243 Take precautionary measures against static discharges

In the event of fire, cool the endangered containers with water

Advice on general occupational hygiene

Store work clothing separately

Remove soiled or soaked clothing immediately

Follow the usual good standards of occupational hygiene

_____________________________________________________________________________________________________________

7.2	Conditions for safe storage, including any incompatibilities
Keep only original container at a temperature not exceeding 30°C.

Fill the container by approximately 80% only, as oxygen (air) is required for stabilisation.

Keep out of light.

7.3	Specific end uses

Adhesive in application of hoof care products.

_______________________________________________________________________________________________________________

SECTION 8:	Exposure controls/personal protection

  - Control parameters

Occupational exposure limits

-Methyl methacrylate

Long-term exposure limit (8-hour TWA):WEL 50ppm 208mg/m³

Short-term exposure limit (15-minute): WEL 100ppm 416mg/m³

*WEL= Workplace Exposure Limit*

DNEL		Industry- Inhalation: 1.35mg/m³

Industry- Skin contact: 1.19mg/kg

Consumer- Inhalation: 0.34mg/m³

Consumer- Skin contact: 0.29mg/kg

Consumer- Ingestion: 2.37mg/kg

PNEC		Fresh water; 0.153mg/l

Marine water; 0.0153mg/l

Soil; 18.68mg/kg dry weight

Fresh water sediment; 45.38mg/kg dry weight

Marine sediment; 45.38mg/kg dry weight

STP: 4.29mg/l

-N,N-Dimethyl-p-toluidine

DNEL		Industry- Inhalation; Long term: 210mg/m³

Industry- Dermal; Long term: 13.67mg/kg/day

Industry- Dermal; Short term:1500mg/m³

Consumer- Inhalation; Long term: 74.3mg/m³

Consumer- Dermal; Long term: 8.2mg/kg/day

Consumer- Dermal; Short term: 1500mg/m³

PNEC		Fresh water; 0.94mg/l

Marine water; 0.094mg/l

Soil; 1.47mg/kg

Sediment; 5.74mg/kg

STP: 10mg/l

- 2-Hydroxyethyl methacrylate

Data lacking.

______________________________________________________________________________________________________________

8.2	Exposure controls

Protective measures		Do not inhale vapours

Avoid contact with eyes

Hygiene measures		Store work clothing separately

Remove soiled or soaked clothing immediately

Follow the usual good standards of occupational hygiene

Respiratory Protection	If ventilation insufficient, wear respiratory protection

Short term- filter apparatus, Filter A

Hand protection		Rubber gloves

Eye protection			Goggles

Body protection		When handling larger quantities: face mask, rubber boots and rubber apron

__________________________________________________________________________________

SECTION 9.	Physical and chemical properties

9.1	Information on basic physical and chemical properties

Physical state: Liquid

Colour: Colourless

Odour: Strong

Freezing point: below -25◦C

Boiling point: Not available

pH: Not available

Evaporation rate:

Flammability: Highly flammable

Upper/ lower flammability or explosive limits: Not available

Vapour pressure: Not available

Vapour density: Not available

Relative density: Not available

Solubility: Not available

Partition coefficient: Not available

Auto-ignition temperature: Not available

Decomposition temperature: Not available

Viscosity: Not available

Explosive properties: Not available

Oxidising properties: Not available

___________________________________________________________________________________

9.2	Other information:

Flash Point: +8oC

Sustaining Combustion: Not available

_______________________________________________________________________________________________________________

SECTION 10:	Stability and reactivity

10.1	Reactivity

Stable under normal conditions.

The following materials may react with the product: Acids. Azo, diazo, hydrazine comps. Alkalis. Oxidising materials.

10.2	Chemical stability

Stable at normal ambient temperatures and when used as recommended

10.3	Possibility of hazardous reactions

Polymerisation with heat evolution may occur in the presence of radical forming substances (eg peroxides), reducing substances, and/or heavy metal ions.

10.4	Conditions to avoid:

Avoid excessive heat for prolonged periods of time. Avoid flames and other sources of ignition.

10.5	Incompatible materials:

Materials to avoid: strong alkalis, strong acids, peroxides, strong oxidising agents.

10.6	Hazardous decomposition products:

Carbon oxides. Nitrogen oxides.

_______________________________________________________________________________________________________________

SECTION 11:	Toxicological information

11.1	Information on toxicological effects

Inhalation: Vapours may irritate throat/ respiratory system. Symptoms following over exposure may include: headache, dizziness, drowsiness.

Ingestion: Gastrointestinal symptoms, including upset stomach.

Skin contact: Irritating to skin. May cause sensitisation by skin contact. Repeated exposure may cause skin dryness or cracking.

Eye contact: Irritating to eyes.

Target organs: Eyes, skin, respiratory system, lungs.

Acute toxicity- dermal (LD50mg/kg): data lacking

Acute toxicity- inhalation (LC50vapours mg/l): data lacking

Skin corrosion/ irritation (animal data): data lacking

Serious eye damage/ irritation: data lacking

Respiratory sensitisation: data lacking

Skin sensitisation: data lacking

Germ cell mutagenicity: data lacking

Carcinogenicity: data lacking

Reproductive toxicity: data lacking

Specific target organ toxicity- single exposure: data lacking

Specific target organ toxicity- repeated exposure: data lacking

Aspiration hazard: data lacking

______________________________________________________________________________________________________________

SECTION 12:	Ecological information

An environmental hazard cannot be excluded in the event of unprofessional handling or disposal.

Harmful to aquatic life with long lasting effects.

12.1	Toxicity:

-Methyl methacrylate

Acute toxicity- fish: LC50 130 mg/l. Exposure time 96h. Species: Pimephales promelas (fathead minnow).

Acute toxicity- aquatic invertebrates: EC50 69 mg/l. Exposure time 48h. Species: Daphnia magna.

Acute toxicity- aquatic plants: 170 mg/l. Exposure time 96h. Species: Selenastrum capricornatum.

-N,N-Dimethyl-p-toluidine

Acute aquatic toxicity: Harmful to aquatic life

Chronic aquatic toxicity: Harmful to aquatic life with long lasting effects

Toxicity to fish: LC50: 46-52 mg/l. Exposure time 96h. Species: Pimephales promelas (fathead minnow).

- 2-Hydroxyethyl methacrylate

Data lacking.

__________________________________________________________________________________________________

12.2	Persistence and degradability

-Methyl methacrylate

Product is readily biodegradable

-N,N-Dimethyl-p-toluidine

No information available

- 2-Hydroxyethyl methacrylate

No information available

_______________________________________________________________________________________________________________

12.3	Bioaccumulative potential

-Methyl methacrylate

Bioaccumulation is unlikely to be significant because of the low water-solubility of this product.

-N,N-Dimethyl-p-toluidine

No information available

- 2-Hydroxyethyl methacrylate

No information available

12.4	Mobility in soil

-Methyl methacrylate

The product has poor water-solubility.

-N,N-Dimethyl-p-toluidine

No information available

- 2-Hydroxyethyl methacrylate

No information available

12.5	Results of PBT and vPvB assessment

-Methyl methacrylate

This product does not contain any substances classified as PBT or vPvB.

-N,N-Dimethyl-p-toluidine

This product does not contain any substances classified as PBT or vPvB.

- 2-Hydroxyethyl methacrylate

This product does not contain any substances classified as PBT or vPvB.

12.6	Other adverse effects:

Not determined.

_______________________________________________________________________________________________________________

SECTION 13:	Disposal considerations

13.1	Waste treatment methods

Directive 2008/98/EC (Waste Framework Directive)

Hazardous waste according to Directive 2008/98/EC (waste framework directive).

Product

Waste is hazardous. It must be disposed of in accordance with the regulations after consultation of the competent local authorities and the disposal company in a suitable and licensed facility.

Contaminated packaging

Dispose of as unused product.

Other disposal recommendation

Do not discharge into the drains/ surface waters/ ground water.

___________________________________________________________________________________

SECTION 14: Transport information

|  | Land transport (ADR/RID) | Inland waterway transport (ADN) | Sea transport (IMDG) | Air transport (ICAO-TI / IATA-DGR) |
| --- | --- | --- | --- | --- |
| 14.1 UN No. | UN1247 | UN1247 | UN1247 | UN1247 |
| 14.2 UN Proper shipping name | Methyl methacrylate monomer, stabilized | Methyl methacrylate monomer, stabilized | Methyl methacrylate monomer, stabilized | Methyl methacrylate monomer, stabilized |
| 14.3 Transport hazard class(es) | 3 | 3 | 3 | 3 |
| ¦ Hazard label(s) |  |  |  |  |
| 14.4 Packing group | II | II | II | II |
| 14.5 Environmental hazards | No | No | No | No |

14.6	Special precautions for user

None known.

14.7	Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

Additional information

Transported in limited quantities.

______________________________________________________________________________________________________________

SECTION 15:	Regulatory information

15.1	Safety, health and environmental regulations/legislation specific for the substance or mixture

EU regulations

-Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures (as amended).

-Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH) (as amended).

Guidance

-Safety Data Sheets for Substances and Preparations

  - Chemical Safety Assessment

No information available.

______________________________________________________________________________________________________________

SECTION 16:	Other information

16.1	Indication of changes

NOTE: Lines within the margin will indicate significant changes from the previous revision

  - Key literature references and sources for data

Relevant manuals and publications

Existing SDS for substances contained in product

  - Relevant R-, H- and EUH-phrases (number and full text)

H225 Highly flammable liquid and vapour

H301 Toxic if swallowed

H302 Harmful if swallowed

H312 Harmful in contact with skin

H315 Causes skin irritation

H317 May cause an allergic skin reaction

H319 Causes serious eye irritation

H330 Fatal if inhaled

H331 Toxic if inhaled

H332 Harmful if inhaled

H335 May cause respiratory irritation

H351 Suspected of causing cancer

H370 Causes damage to organs

H373 May cause damage to organs through prolonged or repeated exposure

H412 Harmful to aquatic life with long lasting effects

  - Other Information

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification.